# Ainekio - Software & System Discussion

Working deliberation for the software architecture, connection/provisioning design, data budget, and the pre-hardware program mockup. Companion to the Design Discussion and Parts Overview docs. Designs here are expected to change during discussion.

## 1. How the robot works (operating narrative)

- MetaHuman OS starts on the home PC, loads an AI model, and connects to a Cloudflare-served web page. This page/endpoint is the meeting point.

- The robot powers on, connects to WiFi, and dials out to the Cloudflare endpoint using saved credentials (robot id + auth token). It never opens inbound ports.

- Once connected, the operator LLM (constantly thinking, queuing tasks) sends intent-level tasks down: walk around, speak this, make a face expression, look at X.

- The robot streams up: low-FPS video or intermittent images, and microphone audio (subject to the wake/VAD policy below).

- Brain-side services process the streams: Whisper server for speech-to-text, face detection for identifying people, the operator LLM for decisions.

- Nightly, the brain enters a sleep/training service; the robot enters a low-power state. In the morning both wake, the model is smarter, personality accrues over time.

- The whole system must degrade gracefully to a mobile-tethered connection, so data usage is a first-class design constraint.

## 2. Division of labor (who computes what)

| Function | Lives on | Why |
| --- | --- | --- |
| LLM operator, task queue | Brain | Big model, changes often |
| Whisper transcription | Brain | Too heavy for ESP32 |
| Face detection | Brain (on received frames) | Same |
| Nightly training / sleep service | Brain | Same |
| Wake word OR voice activity detection | Robot (proposed - see section 5) | Cuts streaming bandwidth massively |
| Intent-to-servo mapping, pose table | Robot firmware | Safety layer; brain never sends raw servo values in normal mode |
| Canned assets (stock phrases, idle moves, boot sounds) | Robot flash | Works offline, saves bandwidth |
| Failsafe (neutral pose on disconnect) | Robot firmware | Must work with no brain at all |

Architecture rule carried over: intent-level commands only. The operator LLM's task queue maps 1:1 onto protocol v0.2 intents with seq numbers; stale commands are dropped, never queued through a disconnect.

## 3. Credentials & provisioning (the login question)

Decision proposed: store credentials in ESP32 flash using encrypted NVS. Do not use the microSD card.

Reasons:

- NVS (non-volatile storage) is the ESP-IDF-native key-value store in flash, designed exactly for WiFi credentials, tokens, and settings. It supports encryption, survives firmware updates (separate partition), and needs no extra hardware.

- The microSD is currently DEFERRED in the pin map (GPIO38/39/40 went to servo signals). Any plan that requires SD for a core function contradicts the current GPIO budget. SD must remain optional.

- An SD card is removable - the wrong place for an auth token even if it were available.

What gets stored in NVS: WiFi SSID + password, brain/Cloudflare endpoint URL, robot id, auth token, servo calibration, protocol version. All changeable at runtime via the mechanisms below.

Provisioning flow (first boot or after credentials cleared):

- Robot finds no valid credentials, starts a temporary WiFi access point (e.g., Ainekio-Setup) with a captive portal page.

- Phone/laptop connects to the AP, portal page asks for: home WiFi credentials, brain endpoint URL, login/token.

- Robot saves to NVS, reboots, dials out to the brain. Portal never runs again unless triggered (button hold or a reset-credentials intent from an authenticated brain).

This matches the interface-app section of the Design Discussion doc (optional direct setup mode) and is the well-trodden path (ESP-IDF has a provisioning framework).

QR code option, assessed: the robot has a camera, and QR decoding on ESP32-S3 is possible (quirc library), so show the robot a QR containing WiFi + token is genuinely feasible and very much in character. Assessment: keep as a phase-2 charm feature layered on top of NVS storage, not the primary mechanism - camera-based provisioning fails exactly when the camera pipeline is unproven, and the AP portal is the reliable fallback either way. Handshake apps (BLE provisioning via Espressif's app): also viable, adds a dependency on their app; the browser portal is more self-contained.

## 4. Storage map (flash vs microSD)

Correction to carry through all docs: the board has 16 MB flash (not GB) and a microSD slot (card not included, currently pinned out of the build). There is no onboard SSD.

Proposed 16 MB flash partition layout:

- Bootloader + partition table + NVS (credentials, calibration): ~0.5 MB

- OTA app slot A: ~3 MB

- OTA app slot B: ~3 MB (enables over-WiFi firmware updates with rollback - strongly recommended since the USB port requires opening the shell)

- Asset filesystem (LittleFS): ~9 MB for canned audio clips, boot/error sounds, OLED face bitmaps, named pose/animation tables

9 MB of assets is more useful than it sounds: at 16 kHz/16-bit mono, speech is ~32 KB/second, so ~4.5 minutes of stock phrases - dozens of canned responses, chirps, and alert sounds. Compressed (ADPCM or similar) that doubles or triples.

microSD status: remains deferred. If a future revision restores it (pin remap or IO expander), good uses would be: audio/photo logging, larger asset libraries, black-box event recorder. It should never hold credentials or anything required to boot.

Extended buffer idea, assessed: using SD as a network buffer (record while offline, upload later) is plausible but adds real firmware complexity (filesystem wear, corruption handling, catch-up upload logic). Recommend: not in v1. The 8 MB PSRAM already provides generous in-memory buffering for frames and audio chunks in flight.

## 5. Audio policy: wake word, VAD, and who listens

The concern is right: streaming the mic 24/7 costs ~32 KB/s = ~115 MB/hour = ~2.7 GB/day. Fine on home LAN, unacceptable tethered. Also a privacy consideration. Options:

- Option A - wake word on the brain: robot streams continuously, brain runs wake word + Whisper. Simplest firmware, maximum bandwidth. Fine for home LAN only.

- Option B - VAD on the robot: firmware runs simple voice-activity detection (energy/spectral) and only streams when someone is speaking. ~90 percent bandwidth reduction in a quiet room. Cheap to compute.

- Option C - wake word on the robot: ESP32-S3 is explicitly supported by Espressif's ESP-SR / WakeNet framework (this is why the S3 + PSRAM exists). Robot listens locally, streams only after the wake word, stops after silence. Minimum bandwidth, most private, most alive-feeling.

Proposal: implement B first (a few dozen lines, testable immediately), design the audio task so C drops in later. Policy becomes a runtime setting: home profile may stream liberally; tether profile requires wake word/VAD. Brain-side wake word (A) remains available in home profile where bandwidth is free.

## 6. No-polling rule (event-driven design)

Agreed constraint from the Design Discussion doc. Concretely:

- Single persistent WebSocket both directions - the brain pushes tasks the moment the operator queues them; the robot pushes events the moment they happen. Nobody asks are you there yet on a timer loop.

- Heartbeats are silence-triggered only (ping only after 2 s of no traffic), so an active conversation generates zero overhead pings.

- Status telemetry (battery, RSSI) rides at 5 s intervals but is a single tiny JSON - and can stretch to 30-60 s in tether/idle profiles.

- The operator LLM thinking loop runs on the brain where CPU is cheap; the robot only reacts to messages and sensor interrupts (I2S DMA, timer-driven servo updates), never busy-waits.

## 7. Power/behavior states (robot side)

Maps the sleep service and data budget onto firmware states:

| State | Camera | Mic | Servos | Network | Entered by |
| --- | --- | --- | --- | --- | --- |
| Active | on (5-10 fps) | streaming | live | full | brain intent |
| Idle | snapshot on request | VAD/wake only | live, idle animations from flash | connected | timeout or brain |
| Dozing | off | VAD/wake only | parked | connected, telemetry slowed | brain sleep intent (nightly service) |
| Deep sleep | off | off | parked, rail could be disabled | disconnected; wakes on timer | brain schedule or low battery |
| Failsafe | - | - | neutral | reconnect loop | link loss 1 s |

Note: true ESP32 deep sleep drops WiFi entirely; wake is by timer or button. For the nightly cycle that is fine (sleep at 11pm, timer-wake at 7am, redial the brain). Dozing keeps the connection for wake on command at modest cost.

## 8. Data budget profiles

| Stream | Home profile | Tether profile |
| --- | --- | --- |
| Video | 5-10 fps VGA MJPEG (~0.5-1.5 Mbps) | snapshots on request or on motion; QVGA option |
| Mic | continuous or VAD | wake-word/VAD gated only |
| Speech out | streamed PCM | prefer canned flash clips; stream only novel speech |
| Telemetry | every 5 s | every 30-60 s |
| Face detection | brain samples frames freely | brain requests a frame only when VAD fires or on schedule |

Profile is a runtime setting sent by the brain (it knows what network it is on better than the robot does).

## 9. Program mockup plan (test before parts arrive)

Goal: a working end-to-end software mockup with zero hardware. Three pieces:

- Body emulator (Python, runs anywhere): implements protocol v0.2 exactly as the future firmware will - dials out via WebSocket, sends hello/status/ack/nak, executes intents against a virtual servo model (simple on-screen skeleton or even text/log output), streams the PC webcam as fake robot camera, PC mic as fake INMP441, plays speech audio out the PC speaker. Honors the state machine in section 7.

- Brain stub / interface dashboard: the ESP32S3 interface app from the Design Discussion doc, built now against the emulator - connection status, e-stop, manual intents, calibration mode, camera panel, audio panels, logs. Browser dashboard served by the brain machine.

- MetaHuman OS adapter: a thin module in MetaHuman OS that speaks the same WebSocket protocol to send operator-queued tasks as intents and receive streams (audio to Whisper, frames to face detection).

Payoff: when the Freenove arrives, firmware development is fill in the real drivers behind an already-tested protocol. Every brain-side feature (wake word, Whisper, face detection, operator queue, sleep service) is developed and demonstrated against the emulator first. The emulator remains permanently useful for testing brain changes without powering the robot.

Suggested build order: protocol library + body emulator, then dashboard with e-stop and manual intents, then MetaHuman OS adapter, then audio path (mic to Whisper round trip), then video path, then profiles/states.

## 10. Compatibility flags & coding issues (running list)

- CRITICAL - board variant: cart line 9 does not confirm N16R8. The R8 octal PSRAM is required for camera streaming. Confirm before ordering.

- microSD vs pin map: any feature that assumes SD storage conflicts with the current servo pin assignments. All v1 storage plans use flash (NVS + LittleFS). SD features are contingent on a future remap.

- Flash size sanity: 16 MB total. Dual-OTA + 9 MB assets fits; keep firmware images lean (avoid pulling in unnecessary Arduino libraries if using Arduino-as-IDF-component).

- Servo library trap (restated): any library that silently uses LEDC will fight the camera XCLK. Servo driver must be written on MCPWM directly.

- I2S full duplex: INMP441 (in) and MAX98357A (out) share clock lines; ESP32-S3 I2S supports full duplex on one controller but mixing sample rates or sloppy DMA config causes glitches. Plan: one I2S peripheral, 16 kHz both directions, DMA both ways. This is a known-fiddly area - allocate bench time.

- Boot-strap pins: GPIO3 and GPIO46 carry servo signals in the candidate map; servos plugged in during boot could interfere with strapping. Bench-test flag already in Design Discussion; firmware should also delay servo attach until after boot completes.

- ADC sampling: battery reads must be averaged and ideally sampled between servo moves; raw reads during stall will false-trigger the 7.0 V warning.

- Cloudflare relay: WebSockets are supported on Workers, but there are connection-duration and message-size limits on free tier; the relay design should assume periodic reconnects are normal (seq/epoch design already tolerates this). Local-LAN direct connection remains the primary mode; relay is for away-from-home.

- Parts list notes: second five-pack of MINI560s unnecessary; standalone BMS boards redundant with protected pack (do not install two protection layers in series without reason); no 20 AWG wire in cart - either add some for the battery-to-switch-to-buck spine or double-up 22 AWG conductors and keep that run short; confirm capacitor assortment includes 1000 uF at 10 V+ and 0.1 uF ceramic (assortment listed is electrolytic - the 0.1 uF ADC cap should be ceramic, verify or add); confirm screw assortment includes ~40 M2 x 5 mm.

## 11. Open questions

- Wake word choice for Option C: Espressif stock wake words vs custom-trained (custom requires their training pipeline; stock words like Hi ESP work out of the box).

- Where does the boredom service live - purely brain-side (sends idle intents) or partly firmware (idle animations from flash when no command for N minutes)? Leaning: firmware idle animations, brain-side boredom intelligence.

- Auth token rotation: does MetaHuman OS issue long-lived tokens or should the robot support re-auth flow? V1 assumption: long-lived token, changeable via dashboard.

- Does the operator LLM need task acknowledgment beyond ack/nak (e.g., task progress, completion events for long intents like walk around)? Probably yes: add {"t":"done","seq":N} to protocol v0.3.

- Emulator fidelity: is webcam-as-robot-camera good enough, or should the emulator also simulate bandwidth constraints (tether profile testing)? Bandwidth throttling flag would be cheap to add.