# Ainekio - System Specification v0.3

STATUS: DRAFT FOR PEER REVIEW. This document is the normative, self-contained specification for the Ainekio robot system. It is written to be implementable by a developer or coding agent with no access to prior design conversations. Rationale and deliberation live in the companion docs (Design Discussion; Software & System Discussion v2; Parts Overview); where this spec and those docs disagree, this spec wins. Keywords MUST, MUST NOT, SHOULD, MAY are used in the RFC-2119 sense.

## 1. Scope and system summary

Ainekio is a desk-companion robot with a distributed architecture. A remote master brain (Python, home PC) runs the AI: an operator LLM (via MetaHuman OS) that queues tasks, a Whisper speech-to-text service, and face detection. The robot body contains a Freenove ESP32-S3-WROOM CAM (N16R8) running C firmware on FreeRTOS that: streams camera frames and microphone audio up, executes intent-level commands down, drives 8 servos and an OLED face, plays speech through an I2S amplifier, and enforces all safety rules locally. The two sides communicate over exactly one WebSocket connection, always initiated by the robot.

Components to be built, in order:

- B1. Protocol library (Python) - message encode/decode/validate for everything in section 3.

- B2. Body emulator (Python) - protocol-faithful fake robot (section 6.3).

- B3. Robot gateway (Python) - WebSocket server, auth, routing (section 6.1).

- B4. Dashboard (browser, served by gateway) - section 6.2.

- B5. MetaHuman OS adapter (Python) - operator tasks to intents.

- B6. Firmware (C, ESP-IDF) - section 5. Built last, against the already-tested protocol.

## 2. Definitions

- BRAIN: the master computer and all its services. BODY: the robot / its firmware / the emulator standing in for it.

- INTENT: a named high-level command (sit, look, speak). The body owns the mapping from intent to servo positions.

- SESSION: one WebSocket connection lifetime. EPOCH: a monotonically increasing integer, incremented by the gateway at every new session.

- PROFILE: a named bandwidth policy (home, tether), set by the brain.

## 3. Protocol specification (v0.3)

### 3.1 Transport

- One WebSocket connection. The body MUST initiate it (dial-out) to the endpoint URL stored in its settings. The body MUST NOT listen on any inbound port except during provisioning (5.8).

- Endpoint MAY be a LAN address (primary) or a relay (e.g., Cloudflare Worker). The protocol is identical in both cases; the relay is a transparent pipe. Implementations MUST treat unexpected disconnect + redial as a normal event, not an error.

- TLS (wss://) SHOULD be used for relay connections; MAY be plain ws:// on the home LAN.

- Text frames carry exactly one JSON object. Binary frames carry media with a 1-byte type header (3.4).

### 3.2 Session establishment

- Body connects and MUST send hello as its first message: {"t":"hello","ver":3,"fw":"","id":"","auth":""}

- Gateway validates the token. On failure it MUST send {"t":"err","code":"auth"} and close. On success it MUST reply: {"t":"welcome","ver":3,"epoch":,"profile":"home"|"tether"}

- The body MUST discard any command whose epoch is not the current one. The gateway MUST increment epoch on every new session, so commands queued before a disconnect can never execute after reconnect.

- If ver mismatches, the side with the higher version decides compatibility; if incompatible, send err code "ver" and close.

### 3.3 JSON control messages

Every brain-to-body command carries "seq" (int, monotonic within an epoch) and is implicitly stamped with the current epoch by session context.

Brain to body, normal operation (intent level only):

- {"t":"intent","seq":N,"name":"sit"}

- {"t":"intent","seq":N,"name":"stand"}

- {"t":"intent","seq":N,"name":"neutral"}

- {"t":"intent","seq":N,"name":"look","yaw":<deg -90..90>,"pitch":<deg -45..45>,"ms":}

- {"t":"intent","seq":N,"name":"walk","dir":"fwd"|"back"|"turn_l"|"turn_r","steps":<1..10>}

- {"t":"intent","seq":N,"name":"face","expr":""}

- {"t":"intent","seq":N,"name":"say","asset":""} (streamed speech uses binary 0x10 instead)

- {"t":"stop","seq":N}  E-STOP: interrupt everything, neutral pose, halt audio out. MUST be honored within 100 ms of receipt.

- {"t":"cam","seq":N,"on":bool,"fps":<0..15>,"res":"QVGA"|"VGA"}  fps 0 = snapshot-only mode

- {"t":"snap","seq":N}  request one frame now

- {"t":"mic","seq":N,"on":bool,"gate":"open"|"vad"|"wake"}

- {"t":"profile","seq":N,"name":"home"|"tether"}

- {"t":"state","seq":N,"name":"idle"|"doze"|"sleep","wake_at":"<ISO8601, required for sleep>"}

- {"t":"ping"}

Brain to body, calibration mode only (body MUST reject with nak code "mode" unless calibration is active):

- {"t":"mode","seq":N,"name":"calibrate"|"normal"}

- {"t":"servo","seq":N,"id":<0..7>,"deg":,"ms":}

- {"t":"limits","seq":N,"id":<0..7>,"min":,"max":,"invert":bool,"center":}

- {"t":"pose_save","seq":N,"name":"","servos":[[id,deg],...]}

- {"t":"cal_save","seq":N}  persist all calibration to NVS

Body to brain:

- {"t":"ack","seq":N}  command accepted and started

- {"t":"nak","seq":N,"code":"stale"|"mode"|"unsafe"|"limit"|"unknown"|"busy","msg":""}

- {"t":"done","seq":N}  long-running intent finished (walk, look, say). Instant intents MAY skip done.

- {"t":"status","vbat":,"rssi":,"state":"","uptime":,"heap":}

- {"t":"event","name":"vad_open"|"vad_close"|"wake_word"|"battery_warn"|"battery_cutoff"|"brownout_recovered"|"boot"}

- {"t":"pong"}

Staleness rules (body side): a command MUST be discarded with nak "stale" if its seq is <= the highest seq already executed in this epoch, or if the body has been disconnected since it was sent (epoch mismatch is impossible by construction, but local receive-time older than 2 s after a link stall MUST also be treated as stale).

### 3.4 Binary media frames

Byte 0 = type. Bytes 1..4 = uint32 little-endian sequence/timestamp counter (sender-local, for ordering and loss measurement). Payload follows.

- 0x01 body-to-brain: mic audio. PCM 16 kHz, 16-bit signed LE, mono, exactly 20 ms per frame (640 payload bytes).

- 0x02 body-to-brain: camera frame, complete JPEG.

- 0x10 brain-to-body: speaker audio, same PCM format as 0x01. Receivers MUST tolerate and drop unknown type bytes. Audio frames MUST be prioritized over camera frames when the sender's outbound queue is congested (drop camera frames first).

### 3.5 Liveness and failsafe

- Either side that has sent nothing for 2 s MUST send {"t":"ping"}; the peer MUST answer {"t":"pong"} promptly.

- Body failsafe: if nothing is received for 3 s (any frame counts), the body MUST enter FAILSAFE: move servos to neutral over 300 ms, stop speaker output, close the socket, and begin reconnecting with exponential backoff (1 s initial, x2, 30 s cap, +/-20% jitter).

- Gateway: if nothing is received for 3 s it MUST mark the robot offline and notify the operator/dashboard.

### 3.6 Timing constants (normative summary)

silence-before-ping 2 s; body failsafe timeout 3 s; e-stop reaction <=100 ms; stale window 2 s; reconnect backoff 1 s..30 s x2 jittered; status interval 5 s (home) / 30 s (tether or DOZING); servo update rate 50 Hz; audio frame 20 ms.

## 4. Safety requirements (all enforced in the body, never trusted to the brain)

- S1. The body MUST refuse (nak "unsafe") movement intents when vbat < 7.0 V, except neutral and stop.

- S2. At vbat < 6.8 V (debounced per S5) the body MUST park to neutral, emit event battery_cutoff, play the low-battery canned clip if possible, and enter DEEP-SLEEP.

- S3. The body MUST clamp every servo target to its calibrated [min,max] and nak "limit" anything outside.

- S4. Raw servo commands MUST be rejected outside calibration mode. Calibration mode MUST time out to normal after 10 min without calibration traffic.

- S5. Battery voltage MUST be computed as the mean of >=16 ADC samples taken while no servo motion is in progress; threshold decisions MUST require the condition to hold across 3 consecutive readings.

- S6. Servo outputs MUST NOT be attached/energized until firmware boot is complete (boot-strap pin caution, 5.4).

- S7. E-stop ({"t":"stop"}) MUST preempt any queued or executing motion and MUST be executable from every state except DEEP-SLEEP.

## 5. Body firmware specification (ESP-IDF, C)

### 5.1 Platform

Target: Freenove ESP32-S3-WROOM CAM, N16R8 (16 MB flash, 8 MB octal PSRAM). Framework: ESP-IDF v5.x. Arduino-as-IDF-component MAY be used for non-timing-critical peripherals (OLED). FreeRTOS tasks and queues are the required structure.

### 5.2 Task layout

| Task | Core | Priority | Job |
| --- | --- | --- | --- |
| wifi/websocket (network) | 0 | high | socket I/O, reconnect, outbound queue drain |
| camera | 0 | medium | esp32-camera frames -> JPEG -> outbound queue |
| dispatcher | 0 | high | JSON parse (cJSON), seq/epoch/safety checks, routing, ack/nak |
| motion | 1 | highest | MCPWM 8ch @50 Hz, interpolation, pose table, idle animations, failsafe neutral |
| audio | 1 | high | I2S full duplex DMA, VAD gate, playback mix (stream + LittleFS assets) |
| telemetry | any | low | ADC per S5, RSSI, status JSON, threshold events |
| oled | any | low | face rendering from assets, status screens |

Inter-task communication is by FreeRTOS queues only; no shared mutable globals without a mutex. The state machine (7) is a small module consulted by all tasks; transitions are serialized through the dispatcher.

### 5.3 Servo driver (hard requirement)

All 8 servo outputs MUST be generated by the MCPWM peripheral, never LEDC (the camera XCLK consumes an LEDC channel; libraries that silently allocate LEDC - e.g., ESP32Servo - are forbidden). 50 Hz, 500-2500 us pulse range mapped to calibrated degrees. Motion between poses MUST be time-interpolated (linear acceptable in v1) at the 50 Hz tick.

### 5.4 Pin map (single config header; two variants; select by build flag)

Camera pins (fixed by board): GPIO 4,5,6,7,8,9,10,11,12,13,15,16,17,18. Reserved: GPIO 43/44 (UART programming/logs), GPIO 35-37 (octal PSRAM), GPIO 0 (BOOT button - MUST NOT be used).

MAP_A (no SD - bring-up default): servo0..7 = GPIO 2,3,14,21,38,39,40,46; battery ADC = GPIO 1 (ADC1); I2S BCLK=41, WS=42, mic DIN=19, amp DOUT=20; I2C SDA=47, SCL=48. microSD unavailable.

MAP_B (SD enabled - valid only if GPIO 33/34 are broken out on the physical board; verify on arrival): servo0..7 = GPIO 1,2,14,21,33,34,45,46; battery ADC = GPIO 3 (ADC1); I2S and I2C as MAP_A; SD_MMC 1-bit on GPIO 38(CMD)/39(CLK)/40(D0).

Constraints: battery ADC MUST be on ADC1 (GPIO1-10; ADC2 conflicts with WiFi). GPIO 3, 45, 46 are strapping pins: servo signal lines on them are outputs only and servos MUST be attached only after boot (S6); this MUST be bench-verified with all servos connected before the map is frozen.

### 5.5 Flash partition table (16 MB)

nvs (encrypted) 64 KB minimum for credentials/settings/calibration; otadata; ota_0 app 3 MB; ota_1 app 3 MB; littlefs assets ~9 MB. OTA updates are pushed by the brain; the bootloader MUST roll back on crash-loop.

### 5.6 NVS contents (all runtime-changeable)

wifi ssid/password; endpoint URL; robot id; auth token; per-servo min/max/invert/center; named poses; profile default; SD-enable flag (MAP_B builds).

### 5.7 Storage rules

Nothing required for boot or normal operation may live on the microSD. LittleFS holds canned audio (PCM or ADPCM), OLED face bitmaps, idle animation tables. SD (MAP_B only) MAY hold logs, photo/audio recordings, and overflow assets.

### 5.8 Provisioning

If NVS lacks valid credentials, or on explicit reset (boot with a designated GPIO held, or authenticated reset command): start SoftAP "Ainekio-Setup" with a captive portal collecting WiFi credentials, endpoint URL, and auth token; write to NVS; reboot to normal dial-out. This is the only moment the body ever accepts an inbound connection.

### 5.9 Audio path

One I2S controller, full duplex, 16 kHz 16-bit mono both directions, DMA both ways. Mic chain: DMA chunk -> VAD (v1: energy threshold with hangover; v2: ESP-SR WakeNet wake word) -> if gate open per current mic setting, emit 0x01 frames. Speaker chain: mixer selects streamed 0x10 frames or LittleFS asset playback; underrun plays silence, never blocks.

## 6. Brain-side specifications

### 6.1 Robot gateway

Python (asyncio + websockets or FastAPI). Responsibilities: accept body connection; validate token; assign epoch; expose an internal API (queue_intent, estop, set_profile, request_snap...) to the adapter and dashboard; stamp seq numbers; route ack/nak/done/status/events back to callers; hand 0x01 audio to Whisper, 0x02 frames to face detection/dashboard; forward TTS audio as 0x10. MUST enforce one active body connection per robot id (newest wins, old socket closed). MUST log all commands and events with timestamps.

### 6.2 Dashboard (minimum viable)

Served by the gateway. MUST have: connection status + heartbeat age; E-STOP button (always visible); manual intent buttons; calibration mode UI (per-servo slider, limits, save); camera panel (snapshot + low-fps view); mic level meter and speaker test; battery/RSSI display; scrolling command/event log. Browser-based; no native app.

### 6.3 Body emulator (acceptance stand-in for hardware)

Python program implementing this spec's body behavior completely: dial-out, hello/welcome, seq/epoch/staleness, ack/nak/done, all safety rules S1-S7 (with a simulated battery voltage control), the state machine (7), VAD on the host mic, PC webcam as camera, PC speakers for 0x10 audio, virtual servo model with calibration and pose table (rendered as a simple skeleton view or logged angles). MUST include a bandwidth-throttle option to rehearse the tether profile and a fault-injection menu (drop link, stall, corrupt frame) for testing gateway robustness.

### 6.4 MetaHuman OS adapter

Translates operator task queue entries into gateway API calls; receives transcripts (from Whisper via gateway), face identities, and body events as operator inputs; instructs sleep/wake per the nightly training schedule.

## 7. State machine (body)

| State | Camera | Mic | Servos | Network | Entry |
| --- | --- | --- | --- | --- | --- |
| ACTIVE | per cam setting | per mic setting | live | full | intent/traffic |
| IDLE | snapshot-only | VAD/wake | idle animations (flash) | connected | 60 s without intents, or brain state cmd |
| DOZING | off | VAD/wake | parked | connected, status 30 s | brain state doze |
| DEEP-SLEEP | off | off | parked, detached | none; RTC timer wake -> redial | brain state sleep (wake_at) or S2 |
| FAILSAFE | unchanged | closed | neutral | reconnect loop | 3 s silence (3.5) |

E-stop is honored in every state except DEEP-SLEEP (S7). Wake from DEEP-SLEEP: RTC timer at wake_at, or power cycle.

## 8. Data profiles

home: video up to 10 fps VGA; mic per setting incl. continuous; status 5 s. tether: video snapshot/motion only (QVGA default); mic gate MUST be vad or wake; status 30 s; speaker prefers canned assets. The brain sets the profile; the body enforces the caps (nak "limit" on out-of-profile cam requests).

## 9. Acceptance tests

Protocol/emulator (pre-hardware, all MUST pass):

- A1. Emulator connects, authenticates, wrong token rejected.

- A2. Intent round trip: ack then done; out-of-order seq nak'd stale.

- A3. Kill link mid-motion: emulator hits FAILSAFE neutral within 3 s; on reconnect, pre-disconnect queued command does NOT execute (epoch).

- A4. E-stop interrupts a long walk intent within 100 ms.

- A5. Raw servo command in normal mode nak'd "mode"; accepted in calibration; limits enforced; cal persists across emulator restart.

- A6. Audio: PC mic -> VAD gate -> gateway -> Whisper transcript appears; TTS audio plays from emulator speakers.

- A7. Video: webcam frames appear in dashboard at commanded fps; under throttle, audio frames win and camera frames drop.

- A8. Simulated battery droop: warn event at 7.0 V (movement nak'd), cutoff behavior at 6.8 V, with S5 debounce demonstrated.

- A9. Profile switch to tether enforces caps.

Hardware bring-up (when parts arrive):

- H1. Freenove tutorial sanity: blink, WiFi, camera web server run on the board as delivered.

- H2. Pinout check: GPIO 33/34 presence decides MAP_A vs MAP_B ambition.

- H3. Strapping-pin boot test: full servo rail connected, 20 consecutive clean boots.

- H4. The core concurrency test: camera streaming + full-duplex audio + 8 servos sweeping + WebSocket command traffic simultaneously for 30 min; measure WiFi drops, servo jitter, audio glitches, frame rate, heap headroom.

- H5. Supervised first battery charge per Parts Overview section 6.

- H6. OTA update + deliberate bad-image rollback.

## 10. Out of scope for v0.3 (recorded, not forgotten)

QR-code provisioning via onboard camera; ESP-SR wake word (designed-for, not required in v1); SD logging formats; Cloudflare relay implementation details; phone-as-brain mode; token rotation; protocol compression.

## 11. Known risk register (for reviewers)

R1 I2S full-duplex DMA configuration (historically fiddly on ESP32). R2 WiFi throughput under simultaneous camera+audio+commands (H4 is the gate). R3 Strapping pins 3/45/46 carrying servo signals (H3 is the gate). R4 2.4 GHz congestion/range in-home. R5 MG90S stall transients vs MINI560 regulation (bulk cap sizing). R6 Cloudflare Worker WebSocket duration limits when relay mode is added. Reviewers are asked to attack: protocol edge cases (3.3/3.5), safety completeness (4), the pin maps (5.4), and the acceptance test coverage (9).